# General Device Troubleshooting – Basic Checks

## Purpose

Use this procedure when an existing device is not working, is offline, is not responding, is disconnected, is intermittently failing, or will not power on.

Complete these basic checks before continuing with a system-specific troubleshooting procedure.

## Applies To

- POS terminals

- Receipt and kitchen printers

- Sticky printers

- KDS devices

- BOH terminals

- Displays

- Card readers

- Other store devices

## Do Not Use This Procedure For

Do not use these steps for new installations, configuration changes, menu or price changes, password resets, access requests, or other administrative tasks.

## Safety Warning

Stop troubleshooting and escalate immediately if you notice any of the following:

- Sparks or smoke

- A burning smell

- Excessive heat

- Liquid damage

- Exposed wiring

- A visibly damaged power cable

Do not restart or reconnect the device under these conditions.

## Step-by-Step Troubleshooting

### Step 1 – Confirm the Device Has Power

Verify that the device is receiving power before changing any settings.

- Confirm that the power cable is securely connected to the device.

- Confirm that the other end is connected to the approved power source.

- Check for a power light, active display, fan, startup sound, or another normal power indicator.

- If the device uses a power strip or UPS, confirm that it is switched on and does not display an alarm or fault.

Expected result: The device powers on normally, or the lack of power is clearly identified and documented for escalation.

### Step 2 – Inspect and Reseat the Cables

Make sure every required connection is secure and free from visible damage.

- Check all applicable power, network, USB, display, and peripheral cables.

- Confirm that every cable is fully connected at both ends.

- Check that the cables are not loose, pinched, frayed, sharply bent, or visibly damaged.

- Gently reconnect any loose cable.

- Do not force a connector into a port.

Expected result: All applicable cables are secure, undamaged, and connected to the correct ports.

### Step 3 – Restart the Device

Perform one controlled restart after completing the power and cable checks.

- Save any active work when possible.

- Use the device’s normal restart or shutdown option.

- Allow the device to shut down completely.

- Turn the device back on.

- Wait for the full startup process to finish.

- Do not repeatedly restart the device if it displays a hardware fault or safety warning.

Expected result: The device completes its startup process and returns to its normal ready state.

### Step 4 – Test the Original Issue

Confirm whether the basic checks resolved the exact problem reported by the store.

- Repeat the action that originally failed.

- Verify the device’s current status.

- Record any error message, status light, unusual sound, or unexpected behavior that remains.

- If the issue continues, proceed with the matching system-specific Knowledge Base procedure.

- If no documented procedure exists, use the appropriate contact from the escalation guide.

Expected result: The issue is resolved, or the Help Desk has enough information to continue with the correct procedure or escalation contact.